Appendix D – Geophysics data

Mills, S.E., 2025, The intersection of framework geology and mineral potential assessment in the Gold Hill mining district: Utah Geological Survey Special Study 175, 54 p., 4 appendices, https://doi.org/10.34191/SS-175.

Summary

The raw geophysics data for Appendix D are contained in two folders, one each for the aeromagnetic and aeroradiometric surveys conducted by Newmont over the Gold Hill region. Additionally, the original grid magnetic and radiometric data are provided in the respective folders. The geophysical data were contributed to this study by Newmont, who flew the survey in 2015 as part of a research and development test of a new in-house airborne system during their exploration in Gold Hill. Line spacing for the survey was 650 ft (200 m) and the lines were flown east-west, perpendicular to the prevailing structural fabric of the district. Aeromagnetic data was flown at a clearance of 200 ft (60 m) and aeroradiometric data were flown at 275 ft (85 m) clearance. Base station location is not known, nor are details of any specific calibrations. The explanation of data fields below is based on the best available interpretation of the data; the data are presented as-is since further technical details of the surveys are not available.

Explanation of data fields

Foler name: Appendix D - Magnetic data

File name: Appendix D - Aeromagnetics data.csv

| Header | Description | Units |
| --- | --- | --- |
| Line | Flight line identifier | Alpha-numeric (unitless) |
| UTM_X | Easting, WGS84 zone 12N | Meters |
| UTM_Y | Northing, WGS84 zone 12N | Meters |
| Latitude | Latitude in WGS84 | Decimal degrees |
| Longitude | Longitude in WGS84 | Decimal degrees |
| GPS_Seconds | Time during flight | GPS seconds |
| GPS_Elevation | Elevation of reading | Meters |
| Height | Filtered height measurement | Meters |
| Raw_Mag | Raw magnetic reading | nanoTesla [nT] |
| Base_Mag | Base station reading | nanoTesla [nT] |
| Corrected_Mag | Raw_Mag - Base_Mag - IGRF | nanoTesla [nT] |
| Masked_Mag | ¦4th_Diff¦ < 0.02 | nanoTesla [nT] |
| IGRF | International Geomagnetic Reference Field correction to apply | nanoTesla [nT] |
| Incl | Magnetic inclination | Degrees |
| Decl | Declination | Degrees |
| TotalField | Residual magnetic Total Field intensity | nanoTesla [nT] |
| Fourth_Diff | Fourth difference, a computed parameter monitoring instrument noise | nanoTesla [nT] |

Folder name: Appendix D – Radiometric data

File name: Appendix D - Radiometric data.csv

| Header | Description | Units |
| --- | --- | --- |
| Line | Flight line identifier | Alpha-numeric (unitless) |
| UTM_X | Easting, WGS84 zone 12N | Meters |
| UTM_Y | Northing, WGS84 zone 12N | Meters |
| GPS_Seconds | Time during flight | GPS seconds |
| GPS_Elevation | Elevation of reading | Decimal degrees |
| Height | Filtered height measurement | GPS seconds |
| Pressure | Air pressure | mmHg |
| Cosmic_Count | Cosmic background spectrum | Counts per second |
| Temperature | Air temperature | Degrees Celsius |
| Up_Comp | Upward uranium compensated | Counts per second |
| Up_Uranium | Upward uranium raw counts | Counts per second |
| TC_Comp | Total counts compensated | Counts per second |
| TC_Count | Total raw counts | Counts per second |
| TC_Lev | Total counts micro-leveled | Counts per second |
| K_Comp | Potassium compensated | Counts per second |
| K_Count | Potassium raw counts | Counts per second |
| K_Lev | Potassium micro-leveled | Counts per second |
| Th_Comp | Thorium compensated | Counts per second |
| Th_Count | Thorium raw counts | Counts per second |
| Th_Lev | Thorium micro-leveled | Counts per second |
| U_Comp | Uranium compensated | Counts per second |
| U_Count | Uranium raw counts | Counts per second |
| U_Lev | Uranium micro-leveled | Counts per second |
| Th_K_ratio | Thorium to potassium ratio | Numeric |
| U_K_ratio | Uranium to potassium ratio | Numeric |
| U_Th_ratio | Uranium to thorium ratio | Numeric |